Downloading LiCor Data to a PC

1.  Turn on the PC and select MS-Prompt from the program menu.

2.  Attach the LiCor to Comm port one of the PC using the 25-pin connecting cable.

3.  At the C:\windows> prompt, type cd..

4.  At the C:\> prompt type licor

5.  At the licor menu type Comm

Creating file

1.  Press the F6 button.

2.  Enter a distinctive file name

Dumping data

1.  Press the Out Key

2.  The display should say "Baud=48", if not press a key to toggle to that display and press enter

3.  Display is "Form=H", press enter

4.  Toggle to "Dump all=No" and press enter

5.  Select the Date and Hour to start the data download

6.  Select the Date and Hour to end the data download

7.  The data should appear on the PC screen.

8.  Press Alt X to exit.

9.  Type cd.. to exit from the licor directory

10.  Type exit to return to Windows

Viewing the Data in Excel

1.  Open Excel.  Then open the file.

2.  The file type is delimited.  That should be selected on the pop-up screen.  Press next.

3.  The Delimiter selected should be space. Press next.

4.  Press Finish.

5.  Data file should open and be ready to manipulate.