Uploading YSI Data to a PC

1.  Turn on the PC.

2.  Attach the YSI to Comm port 1 of the PC using the 9-pin connecting cable.

3.  Open the Ecowatch program

(Click this icon on the desk top )

4.  From the Comm menu (or heading) choose the Sonde option.

5.  In the pop up window titled "Select Com Port", make sure com1 is selected and select OK.

6.  A new window should open up.

Uploading data

1.  Turn the YSI datalogger on

2.  Select "File" from the list given and push the enter button ( )

3.  Select "Upload to PC" from the list given and push the enter button ( )

4.  Select the data file from the list and push the enter button.

5.  A file transfer popup window should appear on the PC screen and a count should start indicating the data is being uploaded.

Viewing and Exporting the Data in Ecowatch

1.  Close the Sonde popup window

2.  From the "File" menu, select "Open"

3.  Select the file just downloaded from the Sonde.

4.  Once the file is open, from the "File" menu, select "Export" and then CDF/WMF

5.  Export this file as a cdf file.  You can rename the file here to be more descriptive.  There is a limit of 8 characters for the file name.

6.  Now close Ecowatch and open Excel

Opening the Data in Excel

1.  Click “Open File” in Excel. To find your exported file, look in the ecowwin folder on the C:\ drive and can be viewed by selecting "all files" from the "Files of Type" drop-down menu at the bottom of the open file window. Select the version that is a "Channel File".

2.  The file type is delimited.  That should be selected on the pop-up screen.  Press next.

3.  Select comma only as the delimiter.  You may need to uncheck the Tab box. Press Next.

4.  Press Finish.

5.  Data file should open and be ready to manipulate.

Manipulating the Data in Excel

1.  Open Multiprobe Template found in on the “Data” page of Field Limn

2.  From the downloaded file, copy and paste the values only of the following columns into the correspondingly titled columns in the template worksheet:

1. Column A	2. Cols. C to F	3. Cols. H to J	4. Cols. K to L	5.Column M

3.  Click and drag the formula in columns J and M in the template to the bottom.

4.  Hit the F12 button to save the manipulated template as its own document with an individual title.

5.  Data is now ready to manipulate.